Aerial Work Platform

- As with other powered vehicles, inspect the lift prior to utilizing it. Walk around and ensure there are no leaks, check to ensure all the controls are functioning correctly.

- Never walk under the boom to gain access to the platform.

- Only utilize the lift on level ground.

- Only stand on the platform floor. Never stand or sit on the railing.

- Always look in the direction the machine is moving.

- Do not rest the boom or basket on a steel structure of any kind.

- Wear safety harnesses and tie-off to the manufacture anchorage point within the platform at all times when you're in the basket. This includes when you are lowered and moving the equipment.

- Keep your hands the external portion of the basket when raising or lowering the basket.

- Ensure that a fire extinguisher is mounted in the basket when performing activities that present a fire hazard such as welding or grinding. Ensure you have a fire watch person below.

- Except in a case of an emergency, ground controls shall not be operated on an occupied lift (lift occupant shall be in full control of the lift at all times).