Carpentry

- Personal Protective Equipment: with Hard Hat, Safety Glasses, High Visibility Shirt, Work Pants, Safety Boots at a minimum.

- Fall Protection: with Full Body Harness with suitable Lanyard and Safety Rails on inspected scaffolding.

- Power Tools and Equipment: to checked for proper working guards.

- Silica Protection: with face shields, dust masks and/or respirators.

- Impalement Protection: with Rebar Caps for Vertical Bars, 2x4 for Horizontal Bars.

- Limited Access Zones (LAZ): to be established equal to the height of the wall plus 4 feet.

- Brace Unsupported Walls: that are 8 feet tall or higher.

- Lifting Inserts: to be verified to be 4 times the maximum intended load applied or transmitted to them as well as crane and rigging requirements.

- Qualified Persons: to be authorized for formwork removal, jack removal, permanent connections, construction load placements and heavy equipment operations.

- Struck By Hazards: to be minimized with delineators for construction equipment traffic.

| 1 | 6 |
| --- | --- |
| 2 | 7 |
| 3 | 8 |
| 4 | 9 |
| 5 | 10 |