SEVERE WEATHER PLAN

REV. 1

This Page Intentionally Left Blank

| page 2

SEVERE WEATHER PLAN

REV. 1

The purpose of the Severe Weather Preparedness Plan (SWPP) is to establish emergency management procedures and guidelines to follow in the event of a severe storm or hurricane.

The plan is separated into three major sections:

1. Section One: procedures for the pre-planning phase during calm weather conditions 2. Section Two: procedures for a hurricane threat

3. Section Three: procedures for recovery activities

Procedures will be coordinated with the project owner and subcontractors.

Areas of Responsibility

1. Management and Supervision:

a. The Project Executive shall provide the overall direction, as required, for the carrying out of this preparedness plan.

b. The Project General Superintendent or the acting Lead Superintendent shall provide guidance over field preparations for subcontractors, visitors, and vendors.

c. Superintendents shall implement actions necessary for the protection of the jobsite, materials, and equipment in their respective areas prior to a storm. General Superintendent is responsible to account for all personnel in their areas prior to evacuation.

d. The Safety Manager shall prepare and update as necessary the SWPP. The plan shall describe in detail the actions necessary to protect the jobsite, materials, equipment, and employees from damage or injury due to inclement weather. The Safety Manager shall coordinate the preparation and evacuation status of the job site. He shall also monitor weather and news reports for updated information.

2. Subcontractors:

a. All supervisors of subcontractors shall take direction from Project Superintendent/ Designee in the event preparations are necessary for an impending storm.

b. Information and direction shall be issued promptly subcontractors.

c. All foremen of subcontractors and craft personnel shall take direction from their immediate supervisors in order to anticipate and prepare for a storm.

3. Office Personnel:

a. All office personnel shall, at the direction of the Project Executive, prepare to leave their offices. Subcontractors shall clear out their offices as they complete preparation work in the field. b. Site Office shall have one person in charge of coordinating document protection activities (files, records, equipment, electronics, etc.)

c. Prior to an office becoming vacated, all subcontractor representatives shall report to the Safety Representative/Competent Person at the project office.

| page 3

SEVERE WEATHER PLAN

REV. 1

Definitions

▪ Essential Personnel – Personnel that will remain on island for the duration of the Hurricane and participate in the Hurricane Aftermath recovery phase.

▪ Non-Essential Personnel – At the first indication of a hurricane within the area (possible threat within 72-48 hours), personnel that will not be needed for the duration of the Hurricane and the Hurricane Aftermath recovery activities will be evacuated.

▪ Gale – Wind speeds of 39-54 mph (34 to 47 knots).

▪ Hurricane Season – June 1st through November 30th as defined by NOAA for the Atlantic, Caribbean, and Gulf of Mexico.

▪ Hurricane – A tropical cyclone with surface winds in excess of 74 mph in the western hemisphere. ▪ Hurricane Watch – A Hurricane condition poses a possible threat, generally within 36 hours.

▪ Hurricane Warning – Hurricane conditions are expected within 24 hours or less. A Hurricane Warning may remain in effect if dangerously high water or exceptionally high waves continue, even though winds may be less than hurricane force.

Hurricane Categories

▪ Category 1 – Minimal Damage Wind speed 74-95 mph, Storm Surge 4-5 feet above normal. ▪ Category 2 – Moderate Damage Wind speed 96-110 mph, Storm Surge 6-8 feet above normal. ▪ Category 3 – Major Damage Wind speed 111-130 mph, Storm Surge 9-12 feet above normal. ▪ Category 4 – Extreme Damage Wind speed 131-155 mph, Storm Surge 13-18 feet above normal.

▪ Category 5 – Catastrophic Wind speed greater than 155 mph, Storm Surge greater than 18 feet above normal.

▪ Storm – A disturbed state of the atmosphere, especially affecting the Earth’s surface, and strongly implying destructive and otherwise unpleasant weather.

▪ Storm Surge – A rise above the normal water level along a shore caused by strong onshore winds and/or reduced atmospheric pressure. A storm surge may even double or more in height when the hurricane’s track causes it to funnel water into a bay. Typically, the stronger the hurricane, the greater the storm surge.

▪ Storm Warning – When winds of 55 mph predicted or occurring, not directly associated with tropical cyclones are expected.

| page 4

SEVERE WEATHER PLAN

REV. 1

▪ Saffir/Simpson Scale – Ratings are normally between 1 and 5 based on the present intensity of a storm (1 being the least severe and 5 being the worst case). This is used to give an estimate of the potential property damage and flooding expected along the coast from a hurricane landfall.

Action Items

Pre-storm (Calm Weather)

1. Supervisors which includes Project Managers and Superintendents are responsible to ensure that the SWPP is in effect and that all subcontractors and personnel are familiar with the established requirement

Note: This plan will be reviewed with the craft employees of all subcontractors through weekly safety meetings.

2. All subcontractors will be supplied with this plan (SWPP). They are required to review this plan with their craft personnel.

Subcontractors will have on file, a list of personnel, inventory of equipment, and list of materials for reference in the event a company representative is not readily accessible during an impending storm. Also, emergency phone numbers shall be provided by subcontractors to (local and/or overseas), should a storm approach during the weekend or holidays and the need to reach a subcontractor arises.

3. Pre-Plan Preparations

a. Establish governmental emergency contact relations if necessary.

i. Develop and maintain a working relationship with emergency personnel.

ii. Provide a symbiotic relationship that can be mutually beneficial.

b. Information sharing includes

i. Supplies

ii. Equipment

iii. Labor

iv. All other items

c. Establishment of Weekly Planning Meetings with Key Personnel Supervision, Subcontractor Superintendents and Project Owner representatives) to discuss the following:

i. Assignment of Responsibilities

ii. Establishment of Chain of Command

iii. Determination of Essential personnel (Identify “Ride out and Recovery Team”) and Non Essential personnel.

iv. Hurricane Preparedness Work Schedule

d. Determination of evacuation procedures for non-essential personnel

i. An Evacuation plan for families and all non-essential personnel shall be developed by each | page 5

SEVERE WEATHER PLAN

REV. 1

project entity in the event of a severe storm with threat to life and property.

e. Mobilization of Materials: Visqueen, Anchor Bolts, Duct Tape, Masking Tape, #9 Tie Wire, Zip Tie Wraps, Plywood, Lumber, Wire Rope/cable Clamps, Nails, Rope, Generators, Pumps and the like. f. Structures within each area of responsibility have been modified to be anchored, secured, and protected.

g. Materials within each area of responsibility have been modified to be anchored, secured and protected.

h. An action plan developed, and responsibilities assigned to essential personnel. i. Construction Material, tools and products

1. Determining storage type, location, and securing procedures.

2. Building materials that are in the field will be placed on pallets and secured on a daily basis.

ii. Heavy Equipment Vehicles

1. Determining storage type, location, and securing procedures.

iii. Hazardous Materials

1. Determining storage type, location, and securing procedures.

i. Office documentation & Equipment protection procedures shall be developed, and responsibilities assigned and designated to essential personnel. Determination of appropriate storage location, Storage Container(s) be located on high ground and properly secured to concrete foundation.

j. Determination of Emergency Gasoline and Diesel Fuel Storage Requirements. Volume amounts, storage type, and site location shall be determined by and approved by the Project Owner.

k. Determination and verification of Potable Drinking water supplies for the project site.

l. Offsite Hurricane Shelter designations for essential personnel. Each project entity is responsible for selecting their appropriate Hurricane Shelter.

i. At least one primary and one secondary Hurricane Shelter shall be identified. Identifying guidelines:

1. On high ground away from highest possible storm surge.

2. Locality to job site.

3. Concrete Structure with hurricane shutters.

4. Adequate size for intended occupants.

5. Adequate amount of Storage for Emergency Supplies.

6. Personal Home used as a Hurricane Shelter.

| page 6

SEVERE WEATHER PLAN

REV. 1

Hurricane Threat Procedures

Alert One (Hurricane Statement issued)

Hurricane Statement issued within the Atlantic Ocean and Caribbean Area by NOAA’s National Weather Service or the Bermuda Weather Service. In general, action taken during this alert consists of major preparation activities.

dck Bermuda, Project Executive will issue a notice to supervisory personnel to review the SWPP and to proceed with storm preparation. The following Action Items will be put into operation. (See Attachment Checklist)

Project Management

1. Review Progress Schedule to determine the storms effect on critical activities. Photos of all site will be taken pre and post severe weather activity to track the damages

2. Meet with appropriate Project Owner’s representative to review and coordinate plans. 3. Begin the process of alerting and reviewing evacuation procedures for non- essential personnel from the island. Non-essential personnel shall be evacuated depending on the time allotment for site removal and their job site role.

Structures on Site

1. New Structures – All structures and parts of structures under construction shall be braced, anchored, and secured as necessary to prevent damages.

2. Office Trailers

a. Office trailers shall be anchored to the ground.

b. Office records and Equipment Removal List - Contract Files, Employee Files, Drawings, Payroll records, computers/modem/discs, printers, submittal records, surveying equipment, fax machines, laser levels, two-way radios,

c. TVs, adding machines, postage meter/scale, phones, progress photos/negatives, first aid kits, and tools/equipment in closet.

d. Designate safe storage location for computer back-ups of jobsite files, specifications, RFI’s, record drawings, etc.

e. Relocate all computers, electronic data storage, copiers, printer, calculators and telephone equipment to a protected area. Cover with plastic to prevent water damage.

3. Storage Trailer and Containers – Storage trailers shall be anchored to the ground with wire rope and locked to prevent wind damage. Containers shall be moved to high ground, anchored, and locked to prevent wind damage.

Utilities

1. Gas – if gas is present on the site, it must be identified, and the valve must be turned off. 2. Electricity – All electrical power not used directly for the project, such as de-watering, shall be disconnected to protect employees from downed power supplies upon return to the job site. 3. Water – Water, when connected to the job site, shall be turned off. When turned back on for use,

| page 7

SEVERE WEATHER PLAN

REV. 1

water shall be tested prior to consumption.

4. Cellular Phones – Phones will be left connected for use in case of emergency, battery levels shall be checked and topped off as necessary.

5. Satellite Phone – Project Satellite phone will be assigned to lead individual and be left connected for use in case of emergency, battery levels shall be checked and topped off as necessary. 6. Clear all storm water drains and prunes trees where a collapse may be possible.

Equipment

1. Crawler cranes, excavators, and dozers will be moved back from the edge of excavation to higher ground and away from anything with the potential of falling onto the machine. The crane booms will be lowered to the ground and the machine will be locked. Masking tape will be placed on all glass to guard against breakage. Service equipment and fueled – especially equipment to be used for cleaning

2. All crane booms and lifts are to be lowered to a safe position. Leads, boom, auger and torque arms should be secured to each other when practical.

3. Earthmoving equipment and on-site vehicles are to be left on high ground with the brakes set. 4. All equipment, such as rubber tire pickers, loaders, backhoes, and form-lifts, will be fueled and moved to higher ground.

5. Welders, compressors, gasoline powered water pumps, and generators will be fueled and moved to higher ground.

6. Gang boxes shall be locked and moved to high ground or inside a sea container. 7. All scaffolding is to be secured from movement. Confirm that all boards and plywood are removed and secured.

8. All trash barrels are to be emptied and secured to stable objects. Used 55-gallon steel drums are to be secured if they cannot be removed from the site.

9. Trash dumpsters are to be emptied and removed from the project site.

10. All Site vehicles shall have fuel levels verified and filled prior to a storm event. 11. All Company vehicles shall have fuel levels verified and be filled prior to a storm event.

Tools

1. Electrical tools shall be removed from the field and stored in a container that has been placed on the highest ground and strapped or chained to a pad. Hand tools shall be locked in their regular storage containers.

2. Each craftsman is responsible for their own personal tools which are to be stored in a trailer placed on high ground.

3. All company owned tools shall be stored in their appropriate storage trailers/containers and accounted for prior to evacuation.

4. Gang Boxes shall be locked and moved to high ground or inside a sea container.

Materials

1. Lumber – bundles shall be banded together.

2. Panels – stacks of unused panels or pans shall be banded.

3. Rebar – unconnected walls and columns shall be braced down to prevent damage from falling. | page 8

SEVERE WEATHER PLAN

REV. 1

Otherwise, rebar laid on the ground shall not be moved.

4. Block, water stop, pipe, wire, and conduit (plastic and metal) shall be secured and protected. 5. Pick up all trash and materials that will not have to be put back on the field in the event that the hurricane watch is lifted.

Vendors / Deliveries

Review material and equipment delivery schedules that may be affected by the storm.

Miscellaneous items to protect

These items shall be rounded up and secured during the preparation phase.

1. Fire Extinguisher

2. First Aid Kits

3. Port-o-johns

4. Drinking water jugs. Drinking water containers are to be filled with potable water and placed in a secure container.

5. Fill and secure all fuel storage tanks.

6. Locate and retain portable electric generators.

7. Locate and retain pumps and hoses for water removal.

Personal Vehicles

No personal vehicles shall be left on the job site. Company vehicles may be left on the job site but, if possible, shall be moved to higher ground so that if flooding occurs, there will be no damage to the environment from oils and fuels. Discussions with the rental car owner and dck Bermuda will determine the placement of Personal Vehicles.

Notifications

1. Notify the Project Manager and Project Owner’s Representative when final preparations are completed.

2. Notify the Corporate Safety Office once the above activities are complete.

| page 9

SEVERE WEATHER PLAN

REV. 1

Alert 2 (Hurricane Watch – Storm within 36 hours)

“Hurricane Watch” is issued by the NOAA’s National Weather Service or Bermuda Weather Service. In general, action taken during this alert consists of advance inspection and procedures for the arrival of the storm and evacuation of the project site (See Appendix 5 for Check List).

Documentation

Assign the task of producing photo and video documentation of the site prior to the storm, preparations for the storm, and damages caused by the storm. This documentation is to be considered part of the Critical Files for the site. Ensure tapes are sent office with non-essential personal.

Action Items

▪ Keep unauthorized people off the job site during the preparation phase.

▪ Pick up all trash and materials that will not have to be put back on the field in the event that the hurricane watch is lifted.

▪ Check tie downs on office trailers.

▪ Fill and secure all fuel storage tanks.

▪ Locate and retain portable electric generators.

▪ Locate and retain pumps and hoses for water removal.

▪ Review work and make arrangements for removal of non-fixed equipment and materials from all areas.

▪ Equipment that will be used for clean-up activities will be serviced and refueled. ▪ Check storm drainage systems for tie-ins, blockage, etc.

▪ Check and secure unused materials, tools, and light equipment.

▪ Notify the project manager and Project Owner’s representative when final preparations are completed.

▪ Notify Corporate Safety Office once the above activities are complete.

| page 10

SEVERE WEATHER PLAN

REV. 1

Alert 3 (Hurricane Warning – within 24 hours)

Hurricane force winds and heavy rains are imminent. “Hurricane Warning” is issued by NOAA’s National Weather Service or Bermuda Weather Service. In general, action taken during this alert consists of making final preparations for the arrival of the storm and evacuation of the project site.

Action Items

Supervisory team completes a final inspection of the Job Site.

Dismissal of all Craft

a. Account for all people (make a sweep of the project site).

b. Notify the project manager and owner’s representative when final preparations are completed. c. Notify the Corporate Safety Office once the above activities are complete.

Evacuation (Category 3 Hurricane)

When the site management has determined if approaching storm is updated to a category 3 hurricane or greater and anticipated to pass over the island, all employees will be evacuated. management team will maintain over-site of the project until the remaining team members can return to the project. However, work shall be planned in such a manner that all personnel are evacuated at the earliest possible hour.

| page 11

SEVERE WEATHER PLAN

REV. 1

Recovery Activities

Recovery and Remobilization after the Storm

After the NOAA’s NWS or Bermuda Weather Service broadcasts that the storm has passed through the area and is continuing to move away, Recovery and Restart of Work may commence. During this phase the work being conducted will be damage assessments, cleanup and site mobilization.

Action Items

1. Damage Assessment by Management.

a. Review all areas of the project for water and wind damage. Note potential problems from downed power lines and other damaged utilities.

b. Create an inventory of damaged equipment, business and personal property, records, drawings & etc. Listing the item description, serial numbers, and quantities involved for the claims process.

c. Document damage with a video camcorder or digital still camera.

d. Notify insurance carrier of potential claims. This step is to be completed by the Corporate Risk Management and coordinated with Project Owner Representatives.

2. Damage Assessment Meeting with Management, Subcontractor Superintendent and Owners. a. Develop, Agree and Implement Project Work Order List.

b. Recommended Recover Work items

i. Remove standing water.

ii. In the event of a downed hazard left on site after the storm, it will be reported to the proper agency/authority. (An example of such a hazard would be downed electrical power lines.) iii. Review project with the electrician and turn on the power when it is determined to be safe. iv. Remove temporary protective measures.

v. Cleanup and dispose of damaged materials and debris.

vi. Absolutely no damaged property or materials are to be removed from the project site without authorization of the Project Superintendent.

vii. Work shall progress in the following order:

● Estimate cost of re-work/repair – accompany with photos of each item if possible ● Work with owner to prepare insurance claim

● Re-evaluate project schedule to determine storm impact

● Make job site stable and safe

● Clean up heavy damage and debris

● Co-ordinate return of evacuated personnel

● Work may proceed

viii. Non-Essential employees will be working from the main office in Pittsburgh, Pennsylvania or wherever their temporary safe haven was located, and will provide logistical support for supplies, project recovery and remobilization activities.

| page 12

SEVERE WEATHER PLAN

REV. 1

Site Crisis Management Team

Emergency Contact List

| Name | Title Phone Email |  |
| --- | --- | --- |
| Brendon Harris | Safety Officer 441-591-0292 bharris@dckww.com |  |
| ¶ ¶ |  | ¶ ¶ ¶ ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |
| ¶ |  | ¶ ¶ ¶ |

Client Contact Information:

Emergency/Recovery Services Contact Information

Royal Bermuda Regiment

Telephone: 441-238-1045

| page 13

SEVERE WEATHER PLAN

REV. 1

Bermuda Red Cross

Telephone: 441-236-8253

| page 14

SEVERE WEATHER PLAN

REV. 1

This Page Intentionally Left Blank

| page 15

SEVERE WEATHER PLAN

REV. 1

This Page Intentionally Left Blank

Hurricane Tracking Chart

| page 16